Plant Health AI Pipeline

Jetson Orin Nano — Deployment & Integration Guide

Edge AI Deployment  |  April 2026

| Property | Detail |
| --- | --- |
| Hardware | NVIDIA Jetson Orin Nano 8GB Developer Kit |
| OS | JetPack 6.x (Ubuntu 22.04-based) |
| Primary Model | XGBoost (ONNX Runtime — Step 1 Stress Classifier) |
| Future Models | GRU Forecaster (PyTorch), Recommendation Engine (Step 3) |
| Interface Protocols | I2C, SPI, UART/RS485, USB |
| Document Scope | Hardware setup, sensor wiring, model deployment, ensemble pipeline |

# 1  Jetson Orin Nano Dev Kit — Hardware Overview

## 1.1  What the Dev Kit Consists Of

The Jetson Orin Nano Developer Kit is a two-part system:

- Module (Jetson Orin Nano 8GB) — the compute brain. Contains the 6-core ARM Cortex-A78AE CPU, 1024-core NVIDIA Ampere GPU, 8GB LPDDR5 RAM, and the DLA (Deep Learning Accelerator). This is where all inference runs.

- Carrier Board — exposes all physical I/O: the 40-pin GPIO header, USB ports, CSI camera connectors, M.2 slots, DisplayPort, and the Ethernet jack. Think of it as the Jetson's motherboard.

## 1.2  First-Time Setup

### What You Need

- Jetson Orin Nano 8GB Developer Kit

- MicroSD card (64GB+) or NVMe SSD (recommended — faster)

- USB-C power supply (5V/3A minimum, 9V/2A recommended)

- HDMI or DisplayPort monitor + keyboard/mouse for initial setup

- Host PC (Windows/Linux/Mac) for flashing JetPack

### Flashing JetPack OS

- Download NVIDIA SDK Manager on your host PC from developer.nvidia.com/sdk-manager

- Connect the Jetson to your host PC via USB-C (put it in recovery mode: hold RECOVERY button, press POWER)

- In SDK Manager, select: Target Hardware = Jetson Orin Nano 8GB, JetPack version = 6.x

- Flash the OS to your NVMe SSD (preferred over SD card for read/write speed)

- Complete the Ubuntu first-boot setup wizard on the Jetson

### Post-Flash: Verify CUDA and Python

nvcc --version                    # should show CUDA 12.x

python3 --version                 # should show 3.10+

python3 -c "import torch; print(torch.cuda.is_available())"  # True

## 1.3  Installing Project Dependencies

Run the following on the Jetson after first boot:

sudo apt update && sudo apt install -y python3-pip python3-smbus i2c-tools

pip3 install onnxruntime numpy pandas joblib smbus2 minimalmodbus

# PyTorch — use NVIDIA's JetPack-optimised wheel (not PyPI):

pip3 install torch torchvision torchaudio --index-url https://developer.download.nvidia.com/compute/redist/jp/v60

TIP: Enable I2C on the Jetson if not already active. Run: sudo usermod -aG i2c $USER then reboot. Verify with: sudo i2cdetect -y 1 (should list detected sensor addresses)

# 2  40-Pin GPIO Header — Pin Reference

The Jetson Orin Nano carrier board exposes a 40-pin header compatible in layout with the Raspberry Pi GPIO header. The diagram below shows all 40 pins with their primary functions. Pins relevant to your sensor suite are highlighted in colour.

| 3.3V / 5V Power | Ground | I2C (SDA/SCL) | SPI | UART | PWM | General GPIO |
| --- | --- | --- | --- | --- | --- | --- |

| Pin# | Signal | Function | J | Function | Signal | Pin# |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.3V PWR | Power | ¦¦¦ | Power | 5V PWR | 2 |
| 3 | GPIO02 | I2C1 SDA | ¦¦¦ | Power | 5V PWR | 4 |
| 5 | GPIO03 | I2C1 SCL | ¦¦¦ | Ground | GND | 6 |
| 7 | GPIO04 | GPIO | ¦¦¦ | UART TX | GPIO14 | 8 |
| 9 | GND | Ground | ¦¦¦ | UART RX | GPIO15 | 10 |
| 11 | GPIO17 | GPIO | ¦¦¦ | GPIO | GPIO18 | 12 |
| 13 | GPIO27 | GPIO | ¦¦¦ | Ground | GND | 14 |
| 15 | GPIO22 | GPIO | ¦¦¦ | GPIO | GPIO23 | 16 |
| 17 | 3.3V PWR | Power | ¦¦¦ | GPIO | GPIO24 | 18 |
| 19 | GPIO10 | SPI MOSI | ¦¦¦ | Ground | GND | 20 |
| 21 | GPIO09 | SPI MISO | ¦¦¦ | GPIO | GPIO25 | 22 |
| 23 | GPIO11 | SPI CLK | ¦¦¦ | SPI CE0 | GPIO08 | 24 |
| 25 | GND | Ground | ¦¦¦ | SPI CE1 | GPIO07 | 26 |
| 27 | GPIO00 | I2C0 SDA | ¦¦¦ | I2C0 SCL | GPIO01 | 28 |
| 29 | GPIO05 | GPIO | ¦¦¦ | Ground | GND | 30 |
| 31 | GPIO06 | GPIO | ¦¦¦ | PWM | GPIO12 | 32 |
| 33 | GPIO13 | PWM | ¦¦¦ | Ground | GND | 34 |
| 35 | GPIO19 | SPI MISO | ¦¦¦ | GPIO | GPIO16 | 36 |
| 37 | GPIO26 | GPIO | ¦¦¦ | GPIO | GPIO20 | 38 |
| 39 | GND | Ground | ¦¦¦ | GPIO | GPIO21 | 40 |

KEY PINS FOR THIS PROJECT: Pins 3 & 5 → I2C Bus 1 (SDA / SCL) — connect all I2C sensors here Pins 1 & 17 → 3.3V power rail — power the ADS1115 and low-voltage sensors Pin 4 → 5V power rail — power NPK sensor module (if 5V) Pins 6,9,14,20,25,30,34,39 → Ground (GND) Pin 8 & 10 → UART TX/RX — alternative serial for RS485 adapter (or use USB)

# 3  Sensor Wiring & Python Drivers

All sensors connect to the 40-pin header or USB. The key rule: any sensor with an analog voltage output (pH probe, electrochemical signal) must go through an ADS1115 ADC first, since the Jetson has no onboard ADC.

## 3.1  Sensor–Pin Mapping

| Sensor | Protocol | Jetson Pins | Library | Notes |
| --- | --- | --- | --- | --- |
| BME280 (Temp + Humidity) | I2C | Pins 3,5 + 1 (3.3V) + 6 (GND) | adafruit-circuitpython-bme280 | Default I2C addr 0x76 |
| Capacitive Soil Moisture | I2C | Pins 3,5 + 1 (3.3V) + 6 (GND) | adafruit-circuitpython-seesaw | Adafruit STEMMA QT, addr 0x36 |
| DS18B20 Soil Temperature | 1-Wire | Pin 7 (GPIO4) + 1 + GND | w1thermsensor | Add 4.7kΩ pull-up on data line |
| TSL2591 Light Sensor | I2C | Pins 3,5 + 1 + GND | adafruit-circuitpython-tsl2591 | Addr 0x29, up to 88k lux |
| ADS1115 ADC (16-bit) | I2C | Pins 3,5 + 1 + GND | adafruit-circuitpython-ads1x15 | Addr 0x48 — up to 4 analog inputs |
| pH Probe | Analog → ADS1115 | ADS1115 channel A0 | via ADS1115 | Output 0-3V mV range |
| Electrochemical Signal | Analog → ADS1115 | ADS1115 channel A1 | via ADS1115 | Raw mV — most important real feature |
| NPK Sensor | RS485/Modbus | USB → USB-RS485 adapter | minimalmodbus | Query registers 0x0000–0x0002 |
| Chlorophyll Sensor | I2C or Analog | Pins 3,5 or ADS1115 A2 | Depends on model | SPAD value output |

## 3.2  I2C Sensor Read — Python Example

All I2C sensors share Bus 1 (pins 3 & 5). Install Adafruit libraries and read like this:

import board, busio

import adafruit_bme280.advanced as adafruit_bme280

import adafruit_ads1x15.ads1115 as ADS

from adafruit_ads1x15.analog_in import AnalogIn

i2c = busio.I2C(board.SCL, board.SDA)

bme = adafruit_bme280.Adafruit_BME280_I2C(i2c)

ads = ADS.ADS1115(i2c)

ambient_temp  = bme.temperature        # degrees C

humidity      = bme.relative_humidity  # %

ph_voltage    = AnalogIn(ads, ADS.P0).voltage  # pH probe

echem_voltage = AnalogIn(ads, ADS.P1).voltage  # electrochemical signal (mV)

## 3.3  NPK Sensor — Modbus RS485 Example

import minimalmodbus

npk = minimalmodbus.Instrument('/dev/ttyUSB0', 1)  # port, slave addr

npk.serial.baudrate = 9600

nitrogen   = npk.read_register(0x0000, functioncode=3)  # mg/kg

phosphorus = npk.read_register(0x0001, functioncode=3)

potassium  = npk.read_register(0x0002, functioncode=3)

WARN: The electrochemical signal probe was pure noise in the synthetic dataset (r=0.02). In real hardware, this should be the STRONGEST stress biomarker — plant electrophysiology directly signals cellular stress. Prioritise getting a clean reading from this sensor. Expect it to become the top SHAP feature once you retrain on real data.

# 4  Loading the Step 1 Stress Classifier onto Jetson

## 4.1  Export XGBoost to ONNX (on Training Machine)

Do this once on the machine where training was done (RTX 5060), then transfer the ONNX file to the Jetson.

from skl2onnx import convert_sklearn

from skl2onnx.common.data_types import FloatTensorType

import joblib, onnx

model  = joblib.load('outputs/models/xgboost.joblib')

scaler = joblib.load('outputs/models/scaler.joblib')

# Export XGBoost to ONNX

initial_type = [('float_input', FloatTensorType([None, 13]))]

onnx_model   = convert_sklearn(model, initial_type=initial_type)

onnx.save_model(onnx_model, 'outputs/models/stress_classifier.onnx')

Transfer both files to the Jetson:

scp outputs/models/stress_classifier.onnx  jetson@<IP>:~/plant_ai/models/

scp outputs/models/scaler.joblib           jetson@<IP>:~/plant_ai/models/

## 4.2  Run Inference on Jetson with ONNX Runtime

import onnxruntime as ort

import joblib, numpy as np

scaler  = joblib.load('models/scaler.joblib')

session = ort.InferenceSession('models/stress_classifier.onnx')

FEATURE_COLS = [

'Soil_Moisture', 'Ambient_Temperature', 'Soil_Temperature', 'Humidity',

'Light_Intensity', 'Soil_pH', 'Nitrogen_Level', 'Phosphorus_Level',

'Potassium_Level', 'Chlorophyll_Content', 'Electrochemical_Signal',

'Thermal_Gradient', 'VPD_proxy'

]

def predict_stress(raw_readings: dict) -> str:

# Derive engineered features

raw_readings['Thermal_Gradient'] = (

raw_readings['Ambient_Temperature'] - raw_readings['Soil_Temperature'])

T  = raw_readings['Ambient_Temperature']

RH = raw_readings['Humidity']

raw_readings['VPD_proxy'] = (1 - RH/100) * 0.6108 * np.exp(17.27*T/(T+237.3))

# Build feature vector in correct order

x = np.array([[raw_readings[f] for f in FEATURE_COLS]], dtype=np.float32)

x_scaled = scaler.transform(x).astype(np.float32)

pred = session.run(None, {'float_input': x_scaled})[0][0]

return ['Healthy', 'Moderate Stress', 'High Stress'][int(pred)]

NOTE: The scaler MUST be applied before every inference call. A missing scaler.transform() call produces silent wrong predictions — no error is thrown. The feature order in FEATURE_COLS must match exactly what was used during training.

# 5  Loading Future ML/DL Models — Ensemble Architecture

As Steps 2 and 3 are completed, their models are deployed alongside Step 1 on the Jetson. The final system is an ensemble pipeline where each model feeds the next, and the outputs are combined for a final decision.

## 5.1  Planned Ensemble Pipeline

| Step | Model | Format | Role | Trigger |
| --- | --- | --- | --- | --- |
| Step 1 | XGBoost Stress Classifier | ONNX | Classifies current sensor snapshot | Every reading (every 6h) |
| Step 2 | GRU Stress Forecaster | PyTorch (.pt) | Predicts stress class 6–24h ahead from recent window | When Step 1 = Moderate or High |
| Step 3 | Recommendation Engine | Rule table + optional RF | Suggests intervention (water, fertilise, relocate) | When Step 2 forecasts worsening |
| Ensemble | Soft-voting / Weighted Average | Python logic | Combines Step 1 + Step 2 confidence for final alert | Always |

## 5.2  Loading the Step 2 GRU (PyTorch) on Jetson

PyTorch models are loaded directly from the saved .pt checkpoint:

import torch

checkpoint = torch.load('models/gru_forecaster.pt', map_location='cpu')

model = GRUForecaster(

input_size  = checkpoint['input_size'],

hidden_size = checkpoint['hidden_size'],

num_layers  = checkpoint['num_layers'],

num_classes = 3

)

model.load_state_dict(checkpoint['model_state_dict'])

model.eval()

# Inference: pass a sliding window of recent readings

# window shape: (1, window_size, n_features)

with torch.no_grad():

logits = model(window_tensor)  # shape: (1, 3)

probs  = torch.softmax(logits, dim=1)  # class probabilities

TIP: Use map_location='cpu' when loading on Jetson if the model was saved on a CUDA device. The Jetson Ampere GPU is available but for GRU inference on small windows, CPU is often sufficient. To use the Jetson GPU: map_location='cuda' and model.to('cuda').

## 5.3  Ensemble Inference — Combining Step 1 & Step 2

def ensemble_predict(current_reading, recent_window):

# Step 1: current-state classification

step1_label = predict_stress(current_reading)   # 'Healthy'/'Moderate'/'High'

step1_conf  = get_step1_confidence(current_reading)  # np.array shape (3,)

# Step 2: 24h forecast (only if sensor shows any stress)

if step1_label != 'Healthy':

step2_probs = gru_forecast(recent_window)   # np.array shape (3,)

# Weighted combination: 60% current, 40% forecast

combined = 0.6 * step1_conf + 0.4 * step2_probs

final_class = ['Healthy','Moderate Stress','High Stress'][np.argmax(combined)]

else:

final_class = 'Healthy'

return final_class

## 5.4  Model File Conventions on Jetson

| File | Format | When Available |
| --- | --- | --- |
| models/stress_classifier.onnx | ONNX | Now (Step 1 complete) |
| models/scaler.joblib | joblib | Now (Step 1 complete) |
| models/gru_forecaster.pt | PyTorch checkpoint | After Step 2 training on real data |
| models/recommendation_rules.json | JSON / rule table | After Step 3 is built |
| models/scaler_v2.joblib | joblib | After retraining on real hardware data |

# 6  Next Steps — Project Continuation

## 6.1  Immediate Hardware Tasks (Before Step 2)

- Wire BME280 + Capacitive Soil Moisture sensor to I2C Bus 1 (pins 3, 5). Verify with i2cdetect -y 1.

- Wire ADS1115 ADC to I2C Bus 1. Connect the electrochemical signal probe to channel A1 and pH probe to A0.

- Connect NPK sensor via USB-RS485 adapter. Verify /dev/ttyUSB0 appears.

- Wire DS18B20 soil temperature probe to GPIO4 (pin 7) with a 4.7kΩ pull-up to 3.3V.

- Write a unified sensor_read.py script that collects all channels every 6 hours and appends to a CSV log.

- Run the Step 1 inference on each real reading and log both the raw readings and the predicted label.

## 6.2  Building Step 2 — GRU Stress Forecaster

Step 2 requires temporally correlated real sensor data. The synthetic dataset cannot support it (ACF ≈ 0). Once you have several weeks of real hardware readings:

- Create sliding windows of the last N readings (e.g. N=8 = 48 hours of history) as input sequences.

- Target: the stress class 4 readings (24 hours) ahead.

- Train a GRU with hidden_size=64, num_layers=2 using the same temporal split strategy as Step 1.

- Transfer the trained .pt file to the Jetson and load it as shown in Section 5.2.

## 6.3  Building Step 3 — Recommendation Engine

Step 3 maps a predicted stress state + identified root cause (from SHAP) to a concrete care action:

| Stress State | Primary SHAP Driver | Recommended Action |
| --- | --- | --- |
| High Stress | Soil_Moisture < threshold | Irrigate immediately — critical water deficit |
| High Stress | Nitrogen_Level < threshold | Apply nitrogen fertiliser |
| High Stress | Both low | Full intervention: water + feed |
| Moderate Stress | VPD_proxy high | Increase ambient humidity / reduce temperature |
| Moderate Stress | Soil_Moisture declining | Schedule watering within 12 hours |
| Healthy | — | No action — log reading |

## 6.4  ONNX Export — Retraining Workflow

Each time a model is retrained on new real-hardware data, follow this export checklist:

- Retrain model on updated dataset (training machine).

- Re-fit StandardScaler on new training data — the scaler parameters will differ from synthetic data.

- Export updated XGBoost to ONNX using skl2onnx (same as Section 4.1).

- Save updated scaler as scaler_v2.joblib — never overwrite the original until the new version is validated.

- SCP both files to Jetson. Run a smoke test (10 known readings) before switching production inference.

- For GRU updates: retrain on training machine, save new .pt checkpoint, SCP to Jetson, reload in inference loop.

## 6.5  Project Milestone Tracker

| Milestone | Description | Status |
| --- | --- | --- |
| Step 1 — Classifier | XGBoost trained, F1=0.995, zero dangerous errors | Complete |
| ONNX Export | Export stress_classifier.onnx for edge deployment | Pending |
| Hardware Wiring | All sensors connected and reading on Jetson | Pending |
| Real Data Collection | Minimum 2 weeks of 6-hourly real sensor readings | Pending |
| Scaler Retrain | Refit scaler on real sensor distributions | Pending (after real data) |
| Step 2 — GRU Forecaster | Train on real temporally-correlated data | Pending |
| Step 3 — Recommendation | Build rule table + optional classifier | Pending |
| Ensemble Integration | Combine Step 1 + 2 + 3 in unified inference loop | Pending |
| Dashboard / Alerting | Real-time monitoring UI or notification system | Future |

Plant Health AI Pipeline  |  Jetson Orin Nano Deployment Guide  |  April 2026